Lab Section (Day/Time)__10.13/14:00_____________________

Names of Student with ID#

Student _Li Xianzhe____2022214880____________              Marks ___________

EE 188L - Lab 6

Introduction to Operational Amplifier Circuits

NAU + CQUPT (Fall 2020)

- Introduction

The Operational Amplifier (Op-Amp) is a highly-sophisticated (complicated) solid-state (Transistorsnext semester in EE 280) circuit designed to be used in many applications to make mathematical relationships between input electrical signals and output electrical signals. It is used primarily in instrumentation that produces output voltage signals that are a mathematical function of the input voltage signals [Vout = f(Vin)].

The basic properties of the ideal op-amp have been introduced and discussed in class, along with additional details and results of those basic properties. Some of the results are not obvious at the first time that the student has seen them.

The op-amp has 2 DC voltage inputs that supply DC voltages to it which power the transistor circuits inside—sometimes these are not shown in schematics. The op-amp has 2 signal inputs and 1 signal output. Additional ports (connections from inside the op-amp circuit to outside) may exist for some op-amps, but this lab does not require those.

As explained in class the ports and properties of the ideal op-amp are:

- vp => noninverting input with respect to ground.

- vn => inverting input with respect to ground.

- vd = vp - vn   => difference input voltage (Note: not with respect to ground).

- vo => output with respect to ground.

- Rin = ∞ => input resistance between vp and vn; this results that the currents into the input ports i+ & i- = 0.

- Rout = 0 => output resistance at the vo port

The Rin & Rout values no loading at the inputs or output.

- Avo = ∞ with infinite bandwidth (BW)

- No offset output voltage if vd, vp & vn all = 0, then vo = 0

- And an external negative feedback path.

The last “property” of the ideal op-amp is actually a fundamental of its design, which is its external negative feedback (NFB) circuit path from the output port to its inverting input port (the “secret of the op-amp):

The op-amp senses the voltages at vp and at vn  If vd = vp – vn ever starts to become not = to 0, the op-amp provides whatever it can, through the feedback path, to drive vd  = vp – vn back to zero!

A block diagram of a general op-amp feedback system is shown below. An input circuit R- goes to the inverting input; an input circuit R+ goes to the noninverting input; and a feedback circuit Zf goes between the output and the inverting input, providing the NFB path. `

Vin-     Inverting ckt R-                Feedback ckt Rf

Vin+       Noninverting ckt R+                                                   Vout

A brief Table of these properties of a real Op-Amp is given below. Slew rate and CMRR are additional properties that are not covered at this time. 100 dB = 10E5

- Circuits and equipment

OBJECTIVE:

To become familiar with the use and characteristics of a 741 op-amp as an ideal op-amp, by building and testing the following circuits:

Inverting Amplifier

Non-inverting Amplifier

Summing Inverting Amplifier

- Difference Amplifier

MATERIALS:

1.	741 Op-Amp

2.	Assorted Resistors

3.	Function Generator

4.	Digital Multimeter

5.	Dual Channel Oscilloscope

6. Multiple output DC Power Supply

PROCEDURE:

Comments:

- Use +/- 15 volts for the Op-Amp DC voltages on pins 7 & 4. Follow the instructions on creating +/- voltages!!

- Use 0.01 to 1.0 AC sine wave signals, depending on the circuit amplifier gains.

- The first 3 circuits use the same values for Rin and Rf: thus you will not need to completely rebuild each circuit from the beginning. The last one has given values for the resistors.

- The labels for resistors may be different on the figures. You can figure out which is Rf and which is Rin, etc.

Sample circuit for an inverting amplifier:

Circuits to be built today:

- Inverting Amplifier

- Design a combination of RF and Rin (Note: not the same Rin as the input impedance of the ideal op-amp) for a calculated gain of -10; and record the calculated gainVo/Vi, for the circuit shown below.  Assume an ideal op-amp. Use kΩ ranges.

RF=  10   kΩ and Rin=  1    kΩ

- Construct the circuit shown below, referring to the figure above for the pin-out configuration of the 741 op-amp.

- For f = 100Hz, use a sine wave from the FG. Adjust the input to 0.4Vp_p (peak-to-peak).

- Measure and record voltage gain Vo/Vin . Compare with calculated value. Use peak values. Measured gain=   -10.2   .

- Find the maximum peak-to-peak output voltage without distortion by increasing the input voltage until Vo begins to clip: it should be a bit below the ±15VDC power sources at pins 4 &7.

- Max input Vpp =   2.96V       .    Max output Vpp =   27.6V       .

Rin=__0.99___ k          Rf=__10.1___ k

- How were you able to find that the output is ‘inverted’?

Explain: By looking at the phase of the input and output waveforms in the oscilloscope, a phase difference of 90° can be observed

- Noninverting Amplifier

- Design a combination of RF and R1 for a calculated gain of +11; and record the calculated gainVo/Vi for the circuit shown below.  Assume an ideal op-amp. Use kΩ ranges.

R1= 1    kΩ and RF= 10   kΩ

- Construct the circuit shown below, referring to the figure above for the pin-out configuration of the 741 op-amp.

- For f = 100Hz, use a sine wave from the FG. Adjust the input to 0.4Vp_p (peak-to-peak)

- Measure and record voltage gain Vo/Vin. Compare with calculated value. Measured gain=  11.1    .

- Find the maximum peak-to-peak output voltage without distortion by increasing the input voltage until Vo begins to clip: they should be a bit below the ±15VDC power sources at pins 4 &7.

Max output Vpp =   28.0 V.	  Max input Vpp =    2.72 V   .

R1=__10.1___ k          Rf=__0.99__ k

- How were you able to find that the output is ‘non-inverted’?

Explain: By looking at the phases of the input and output waveforms in the oscilloscope, it can be observed that their phases are the same and there is no phase difference

- Difference Amplifier

- Design the difference amplifier below, which has a difference gain = 10.

Use kΩ range resistors. R1=R3= 1 k , R2=R4=10 k

- Build it and verify the calculated difference gain

Measured gain=  10    .

- Using two channels of the FG, to produce two way input: For f = 100Hz, use a sine wave from the FG. Adjust the input to

V1pp = 0.1 V and V2pp = 0.5 V

- Output voltage Vpp =    4.0V

R1=R3=_10.1__ k          R2=R4=___0.99__ k

- How were you able to find that the output is a ‘difference’?

Explain: It can be observed that the output VPP is 10 times greater than the difference between the two inputs in the oscilloscope.

- Summing Amplifier

- Analyze the summing circuit below and determine the theoretical Vout as a function of the inputs V1, V2 and V3.

theoretical Vout = _583.6mV______

- Build it and verify by measurement the predicted output for V1 = V2 = V3 = 0.01 Volts AC(Prof Du has changed the voltage requirement for these experiments to 0.02V), sine wave, f=100Hz.

Just apply the same voltage to all 3 inputs.

- Record output voltage Vpp =    568mV

- How were you able to find that the output is ‘summed’?

Explain: As can be seen, the output VPP is the weighted sum of the three inputs in the oscilloscope with R4.

Attachments:

Prof Du`s signature:

Results of each experiment oscilloscope(as required by Prof Du)

A：

And its input is distort when maximum:

B:

And its input is distort when maximum:

C:

D: