Lab Section (Day/Time)_______________________

Names of Student with ID#   2022214880

Student _Li Xianzhe________	Instructor signature ______________________

NAU + CQUPT

EE 188 Lab 8

Complex Power in AC Circuits

Introduction:

In passive AC circuits with both reactive and resistive elements (inductors, capacitors and resistors), Power must be complex-valued in order to account for the energy expended in actual work (Watts) and that which is tied-up in the maintenance of expanding and collapsing electric and magnetic fields in the reactive elements (Vars).

Complex Power in sinusoidal signals is best accounted for by using complex-valued notation.

The formula for complex power S in a circuit with a current and voltage at a port is given by

S = ½ VI* (VA)

where V = V/θv is the voltage phasor (V)

I = I/θi is the current phasor (A)

Phase angles /θv and /θi are with respect to a reference 00 elsewhere.

Note that the phasors are peak values and not rms values in this expression. If rms values are used, the ½ factor would already be accounted for. For this exercise, we shall stick with peak values.

In rectangular and polar forms S is also expressed by

S = P + jQ

where P = real power (Watts)

Q = reactive power (Vars)

S = S/θp

where S = apparent power (VA)

θp = power angle = (θv – θi) (electrical degrees)

The factor cos(θp) is defined as the power factor (PF), and is an indication of the amount of available real power (of the apparent power S) that can be delivered or absorbed by the circuit at the input.

Procedure:

Set-up the following circuit, adjusting the function generator to

vs(t) = 0.5 cos(2π500t) volts.

Note that the load is between nodes A and B, which is R2 in parallel with the R3 - C1 branch.

Measure and note the exact resistor and impedance values:

R1 = 2.7kΩ => _____2701.7Ω___________(measured)

R2 = 3.6kΩ => _____3693.3Ω___________(measured)

R3 = 4.3kΩ => _____4288.6Ω___________(measured)

C1 = 0.1µF   ZC = -j/ωC = _____-j3183.1Ω___

Activities A, B & C

- Phasor, Impedance & Power Calculations:

- Calculate ZLoad (show all steps, as done in lectures)

- Calculate ZT.

- Calculate total current IT.

- Using voltage division, calculate VLoad.

- Calculate total complex power ST

- Calculate load complex power SLoad.

- Show that the total complex power equals the load complex power + the real power in resistor R1 (which will also have to be calculated).

- Measurements:

Using both channels of your oscilloscope,

- Measure the input voltage signal and use it as reference 00

phasor Vs = 0.5/00 (volts)

- Measure the load voltage signal:

VL =_0.237∠-7.92∘V (phasor)

and use it to calculate SLoad and ST of part A above.

- Clearly show all your steps and calculations, as done in class!

- Drawing graphs:

Plot on the Complex (Re, Im) plane the 3 separate graphs below:

- All Impedance Vectors, showing that they add vectorally

ZT = ZT/φT = ZLoad + R1

- All units are in Ohms (Ω)

Example – drawing impedances

- The Complex Power Vectors, showing that they add vectorally

ST = SL + PR1 = PT + jQT

- All units are Power units (VA, Vars & Watts)

Example – the power vector

- The Phasor Vectors for VS = 0.5/00 (ref), IT and VL.

Example – drawing phasors

Note: You will have to put in the proper scales and units for x- and y-axis in each of the complex-plane graphs below

Impedance (kΩ)

Complex Power (VA, Watts, Vars)

Phasors (volts & mA)